(QUAKER ”V300 CA” PROJECT-OUT CASEMENT WINDOW)

SECTION 08 53 13 (08560) - VINYL WINDOWS

PART 1	  GENERAL

1.01	SUMMARY

A. 	Section Includes:

1. 	Operating and Fixed Vinyl Window Units

a. Project-out Casement Vinyl Windows.

2. 	Glass and Glazing for Vinyl Windows.

3. 	Wood Blocking, Shims, Anchors, Clips, and all accessories necessary for a complete installation furnished and installed.

4. 	All trim and closure pieces

5.  Installation labor, tools, equipment, and services necessary for installation of Vinyl Windows.

B.	Related Sections:

      - Section 07 62 00 (07620) -  Sheet Metal Flashing and Trim

      - Section 07 92 00 (07920) -  Joint Sealants

      - Section 08 32 16 (08320) -  Vinyl Patio Doors

      - Section 08 41 13 (08410) – Aluminum Entrances and Storefront

      - Section 08 80 00 (08800) - Glazing

- 1.02	REFERENCES

A.	American Architectural Manufacturers Association (AAMA):

      - 101 – “Voluntary Performance Specification for Windows, Skylights and Glass Doors”.

      - 502 – “Voluntary Specification for Field Testing of Newly Installedf Fenestration Products”

      - 615 – “Voluntary Specification, Performance Requirements And Test Procedures For Superior Performing Organic Coatings On Plastic Profiles”

      - 1503 – “Voluntary Test Method for Thermal Transmittance and Condensation Resistance of Windows, Doors and Glazed Wall Sections”

      - 2400 – “Voluntary Specification for Installation of Windows with a Mounting Flange in Stud Frame Construction”

- C.  American National Standards Institute (ANSI) Publications

- Z97.1 – “Performance Specifications and Methods of Test for Safety Glazing Materials Used in Buildings”

- D.  ASTM International (ASTM) Publications:

- C518 – “Standard Test Method for Steady-State Thermal Transmission Properties by Means of the Heat Flow Meter Apparatus”

- C1036 – “Standard Specifications for Flat Glass”

- C1048 – “Standard Specifications for Heat-Treated Flat Glass – Kind HS, Kind FT Coated and Uncoated Glass”

- D3985 – “Standard Test Method for Oxygen Gas Transmission Rate Through Plastic Film and Sheeting Using a Coulometric Sensor”

- E90 – “Standard Test Method for Laboratory Measurement of Airborne Sound Transmission Loss of Building Partitions and Elements”

- E283 – “Standard Test Method for Determining the Rate of Air Leakage Through Exterior Windows, Curtain Walls, and Doors Under Specified Pressure Differences Across the Specimen”

- E330 – “Standard Test Method for Structural Performance of Exterior Windows, Curtain Walls, and Doors By Uniform Static Air Pressure Difference”

- E331 – “Standard Test Method for Water Penetration of Exterior Windows, Curtain Walls, and Doors By Uniform Static Air Pressure Difference”

- E413 – “Classification for Rating Sound Insulation”

- E547 – “Standard Test Method for Water Penetration of Exterior Windows, Skylights, Doors, and Curtain Walls by Cyclic Static Air Pressure Differential”

- E774 – “Standard Specification for Sealed Insulating Glass Units”

- E1886 – “Standard Test Method for Performance of Exterior Windows, Curtain Walls, Doors, and Impact Protective Systems Impacted by Missile(s) and Exposed to Cyclic Pressure Differentials”

- E1996 -  “Standard Specification for Performance of Exterior Windows, Curtain Walls, Doors and Impact Protective Systems Impacted by Windborne Debris in Hurricanes”

- F588 -  “Standard Test Methods for Measuring the Forced Entry Resistance of Window Assemblies, Excluding Glazing Impact”

- F1249 – “Standard Test Method for Water Vapor Transmission Rate Through Plastic Film and Sheeting Using a Modulated Infrared Sensor”

- E.  Glass Association of North America (GANA):

- 1.	“GANA Glazing Manual”

- F.  Federal Specifications (FS) Publications:

- 1.	FS-RR-W-365A “Wire Fabric (Insect Screening)”

- G.  Insulating Glass Certification Council (IGCC)

- H.  Insulating Glass Manufacturers Alliance (IGMA) Publications:

- 1.	Glazing Guidelines

- I.   National Fenestration Ratings Council (NFRC)

- J.  Screen Manufacturers Association (SMA) Publications:

- 1.  	1004 “Specifications for Aluminum Tubular Frame Screens for Windows”

- K.  U.S. Consumer Product Safety Commission (CPSC) Publications:

- 1. 	16 CFR Part 1201 “Safety Standard For Architectural Glazing Materials”

- L.  Window and Door Manufacturers Association (WDMA) Publications:

- 1.	ANSI/AAMA/WDMA  101/I.S.2/NAFS-02 “Voluntary Performance				Specification for Windows, Skylights and Glass Doors”

- 2.  	AAMA/AAMA/WDMA/CSA  101/I.S.2/A440 “Standard/Specification for 			Windows, Doors and Unit Skylights”

1.03	SUBMITTALS

A. 	Submit “Letter of Conformance” in accordance with Section 01 33 00 (01330) – with the following supporting data:

- 1. 	Product data for each type of vinyl window specified, including standard construction details, dimensions of individual components, profiles, finishes, hardware, and accessories.

2. 	Shop drawings for each type of window specified, including ¼-inch scale wall elevations, typical unit elevations at ¾-inch scale details, and full size details of typical composite members.

3.  Samples:  Provide full-size or partial-size sample of window illustrating		glazing system, quality of construction and finish.

4.  Submit certified independent laboratory test reports verifying compliance 	with all test requirements of 1.05 PERFORMANCE				REQUIREMENTS as requested by Architect.

1.04	DEFINITIONS

A. 	Performance grade number, included as part of the AAMA/WDMA/CSA 101/I.S.2/A440 product designation code, is actual design pressure in pounds force per square foot used to determine structural test pressure and water test pressure.

1.05    PERFORMANCE REQUIREMENTS

A.   Certify that windows have been tested in accordance with American			       Architectural Manufacturers Association (AAMA/WDMA) Specification for 		       Performance Class specified complying with the following performance 		       standards:

### AAMA/WDMA/CSA 101/I.S.2/A440 Performance Requirements:  Provide vinyl windows of the performance class and grade indicated that comply with AAMA/WDMA/CSA 101/I.S.2/A440.

- Performance Class:	P-CW

- Performance Grade:	50

- Structural Test Performance Requirements (ASTM E330):

- Uniform Load Deflection Test:  No deflection of any unsupported span L of test unit in excess of L/175 at both a positive and negative load of 50 PSF.

- Uniform Load Structural Test:  Unit to be tested at 75 PSF, both positive and negative, with no glass breakage; damage to make windows inoperable; or permanent deformation of any main frame or ventilating member in excess of 0.2% of its clear span.

- Water Resistance (ASTM E331 and ASTM E547):	No water penetration at test pressure indicated.

- Class P-CW-50: 7.50 PSF

- Air Leakage (ASTM E283):

- Project-out Casement Windows:  Maximum 0.3 CFM per sq./ft. of total exterior surface area, when tested at a static air pressure differential of 1.57 PSF minimum.

- Fixed Windows:  Maximum 0.10 CFM per sq./ft. of total exterior surface area, when tested at a static air pressure difference of 1.6 PSF minimum.

B.  Project Wind Loads:

Please specify design wind pressure, both positive and negative required by the governing building code, as calculated from ANSI A58.1, or as determined by  boundary layer wind tunnel testing.  Minimum for positive and negative pressure = 10.0 p.s.f.

- The system shall be designed to withstand the following loads with respect to the plane of the wall:

- Positive pressure of ___ p.s.f. at non corner zones.

- Negative pressure of ___ p.s.f. at non corner zones.

- Negative pressure of ___ p.s.f. at corner zones.

1.06	QUALITY ASSURANCE

- A.  All window units shall be manufactured by a single source.

- 1.  All windows in any one project must be by the same manufacturer and 	with comparable frame depth, profile, glazing bite, and installation			requirements.  Manufacturer must provide a window system that can

- incorporate all window configurations used on the project.

- 2.  	Standards:  	Requirements for vinyl windows, terminology and 	standard of performance, and fabrication workmanship are those specified 	and recommended in AAMA/WDMA/CSA 101/I.S.2/A440.

- Actual Project Window sizes which do not exceed the “Minimum Test Size” as referred to in Gateway Performance Requirements table in (AAMA/WDMA/CSA 101/I.S.2/A440) shall be tested and certified at or exceeding the actual project maximum size in accordance with all other AAMA/WDMA/CSA 101/I.S.2/A440 requirements.

- Actual Project Window sizes which exceed the “Minimum Test Size” as referred to in the Gateway Performance Requirements table in (AAMA/WDMA/CSA 101/I.S.2/A440) shall be tested and certified at or exceeding the actual project maximum size in accordance with all other (AAMA/WDMA/CSA 101/I.S.2/A440) requirements.

1.07	DELIVERY, STORAGE AND HANDLING

A.  Transportation and Handling:  Transport products by methods to avoid product damage, deliver in undamaged condition in manufacturer’s unopened containers or packaging.  Provide equipment and personnel to handle products by method to prevent soiling or damage.  Promptly inspect shipments to assure that products comply with requirements, quantities are correct, and products are undamaged.

B.  Storage and Protection:  Store products in accordance with manufacturer’s instructions, with seals and labels intact and legible.  Store sensitive products in weather-tight enclosures; maintain with temperature and humidity ranges required by manufacturer’s instruction.

1.08	 WARRANTIES

A.  Vinyl Window Warranty

- Products:  Submit a written warranty, executed by the window manufacturer, for the following:

- Framing, sash components, and hardware:  A period of (1) year from the date of manufacture, against defective materials, finish, and workmanship, including substantial non-compliance with applicable specification requirements and industry standards, which results in premature failure of the windows or parts, outside of normal wear.

- Insulated glass units:  A period of (10) years from the date of manufacture, against insulated glass seal failure unrelated to glass breakage.

- In the event which windows or components are found defective, manufacturer will repair or provide replacements without charge at manufacturer’s option.

- Where applicable, materials which are applied to the face of insulated glass for the purpose of simulating division in glass openings (SDL’s) are warranted against detaching from the glass surface for a period of (5) years.

- Warranty for all components must be direct from the manufacturer (non- pass through) and non- prorated for the entire term.  Warranty must be assignable to the non-residential owner, and transferable to subsequent owners through its length.

- Installation:  Submit a written warranty, executed by the window installer, for a period of (1) year from the date of substantial completion, against defective materials or workmanship, including substantial non-compliance with applicable specification requirements, which result in premature failure.

- In the event which installation of windows or components is found to be defective, installer will repair or provide replacements without charge at the installer’s option.

PART 2	  PRODUCTS

2.01	MANUFACTURERS

A.  Approved Manufacturers:

- Quaker Window Products Company, Inc. (800) 347-0438

- Project-out Casement Window: “V300CA” Series

- Substitutions: Only pre-approved products specified by the Architect will be acceptable.  Submit the following information with proper documentation as required for pre-bid substitution requests, and at least (10) working days prior to bid date.

- Independent test reports certifying that proposed product is in accordance with, and meets all criteria specified in Section 1.05 “PERFORMANCE REQUIREMENTS”.

- Drawing details of elevations and sections, and samples in accordance with, and as specified in Section 1.03 “SUBMITTALS”.

- Copy of manufacturer’s warranty specified in accordance with, and as specified in Section 1.08 “WARRANTIES”.

- Any additional information requested by the Architect.

2.02 	MATERIALS

A.  Vinyl Members:

- All extruded vinyl sections shall be of Polyvinyl (vinyl chloride) (uPVC).

2.03	MANUFACTURED UNITS

A.  Principal window frame members and insert members shall be manufactured		      in accordance with AAMA/WDMA/CSA 101/I.S.2/A440.

B.  Window frame depth shall be 3 1/4” minimum.

C.  Glazing:  Refer to Section 2.05 “GLASS MATERIALS”.

D.  Foam wrap shall be factory applied to the perimeter of window frames for 		      insulation.

E.  Window frames shall be designed to receive snap-on accessory components 		      to exterior.

2.04	COMPONENTS

A.  All fasteners, tools, equipment, and other materials necessary for a complete 		     installation shall be furnished by the Contractor.

B.  Locking handles, cases, and strikes to be stainless steel, or other non-			      corrosive materials compatible with vinyl.

C.  Thermoplastic or thermo-set plastic caps, housings, and other components to		      be injection-molded nylon, extruded PVC, or other suitable compound.

D.  Hardware:

- Hinging Hardware:

- Hinges shall be 4-bar arm type in accordance with (AAMA 904-09) for Heavy Commercial and Architectural Grade Windows.

- Hinge bars shall be constructed of stainless steel, with solid brass shoes, and nylon bearing washers at all pivot points.

- Hinges shall include two friction adjusters per each.

- All hinge components shall be concealed while window is in the closed position.

- Locking Hardware:

- Locks must hold securely up to 200 lbs. of force per lock for negative air pressure and forced entry resistance.

E.  Insect Screens:  Provide interior mounted insect screen panel for each			      operable sash.

Please select one of the following “line 1” items below for screen  type

- Screen Fabric:

- 18 by 18 mesh of 0.012” fabric thickness, .008” yarn diameter.  Comply with FS L-S-125B(4.4.9), Scale 1.

- Screen wire shall contain a durable hydrophobic coating which repels water, and prevents dirt and debris from clinging to the screen

- Screen wire product shall be “Greenguard” certified for low chemical emissions.

- Screen Fabric:  Aluminum:  18 by 16 mesh of 0.013” diameter wire.  Comply with FS-RRW-365, Type VII, except black anodized or “gun metal” coating on wire.

- Screen Frame:  Provide aluminum frames and removable vinyl fabric retainer spline.

- Finish shall match window.

F.  Accessories:

Optional Item

- Safety Device Stops:  For operable windows, provide stops to prevent opening greater than enter opening space measurement.  Stops shall be manufacturer’s standard for intended use, and provided by the manufacturer.

Select one of the Muntin options [ item “a.”] below if required

- Muntins:

- Internal Muntins:

- Roll formed aluminum of specified width, located between glass panes within the sealed insulated glass unit.

- Muntins shall have finish to match color of Window Frame.

- Simulated Divided Lite Muntins:

- Composition shall include:

- Exterior applied extruded aluminum Muntin of specified width, continuously adhered to surface of glass with a high performance acrylic adhesive system.

- Roll formed aluminum of specified width, located between glass panes within the sealed insulated glass unit.

- Interior applied extruded aluminum Muntin of specified width, continuously adhered to surface of glass with a high performance acrylic adhesive system.

- Finish of Exterior and Interior applied muntin components shall comply with Section 2.06 “FINISHES”.

- Finish color of Exterior and Interior applied muntins, and internal muntins shall match Window Frame.

2.05	GLASS MATERIALS:

Please select one of the [ item “A.”] glass types below which is in accordance with design.  The recommended Low-Emissivity (Low-E) coating is an option listed below; however, if the design requires tinted glass, or another specific type of (Low-E), please choose the blank option and add pertinent information

A. Clear Float Glass:  ASTM C1036, Type 1 (transparent glass, flat), 			     Class 1 (clear), Quality q3 (glazing select) as manufactured by one of the 		     following:

- Approved Manufacturers:

- Cardinal Industries

- Approved Substitution by Architect

A.  Coated Low Emissivity Glass:  Type 1 (transparent glass, flat),				      Class 1 (clear), Quality q3 (glazing select), with coating type and 			      performance characteristics complying with requirements specified below:

- Low E Coating:  Surface #2 on insulated units.

- Approved Manufacturers:

- “Loe2-272”; Cardinal Industries

- Approved Substitution by Architect.

A. (Please specify if neither of the two above items are selected)

B. Tempered Glass:  Condition A (uncoated surfaces), Type 1	 		   	     (transparent glass, flat), Class 1 (clear), Quality q3, clear, fully tempered 		      safety glass (meet requirements of ANSI Z97.1).

- All tempered glass shall conform to ASTM C1048, ANSI Z97.1, and CPSC 16 CFR Part 1201.  Tempered glass shall bear permanent monogram indicating tempered quality.  Fabrication marks on tempered glass shall be located to be concealed in completed installation.

C.  Windows shall be glazed as follows:

- Sound Transmission Class (STC) (ASTM E413):  Provide glazing required for conforming to over all STC ratings as specified for vinyl windows.

For sound transmission class requirements, please consult Quaker Window Products (800) 347-0438

D.  Insulating Glass:  Manufacturer’s standard units that comply with specified

quality standards and coatings.

- Provide preassembled units consisting of organically sealed panes of glass enclosing a hermetically sealed dehydrated air space and complying with ASTM E774 for performance classification indicated as well as with other requirements specified for glass characteristics, air space, sealing system, sealant, space material, and desiccants.

- Total Thickness:  1”

- Thickness of Each Pane:  (standard 1/8” or specify)

- Air Space Thickness:  (standard 3/4” or specify)

For higher sound transmission class requirements, changes may be required to the highlighted values.  Please consult Quaker Window Products (800) 347-0438

- Exterior Pane of Glass:

- Provide tempered glass where shown on Drawings and as required by local codes and ordinances.

- Insulated Unit Sealing System:

- Thermal conductivity of insulated glass spacer shall perform in accordance to the following:

- Silicone: 0.202 BTU/hr-ft-F (0.350 W/m-K)

- PIB 0.089 BTU/hr-ft-F (0.155 W/m-K)

- Desiccant (loose fill):  0.017 BTU/hr-ft-F (0.030 W/m-K)

- Spacer:  8.197 BTU/hr-ft-F (14.187 W/m-K)

- Insulated glass unit spacer system must include a secondary dual seal.  This also applied to solid foam warm edge seal glass spacer systems.

2.06	FINISHES

A. 	Finish of Vinyl Components

1. 	Finish of vinyl windows:  Select one:   White   Beige	Sandstone

Optional exterior coatings: select one “item no. 2” below

- Special Exterior Paint Finish:

- Reflective Organic Coating conforming to (AAMA 613, AAMA 614, or AAMA 615), Voluntary Specification, Performance Requirements and Test Procedures.

- Paint shall be applied in three coats to all vinyl exterior frame and sash components after frame construction, as to avoid necessary touch-up painting of welded corners, and be baked on at a temperature of 120 to 130 degrees Fahrenheit.

- Welding, and subsequent touching up of pre-painted extrusions in any frame or sash component shall not be allowed.

- Finish Warranty Period:		10 years from date of manufacture

- Color Selection:  Please select a color

- Acrylic Based Exterior Film:

- Acrylic Base shall be composed of Polymethyl Methacrylate / PMMA with a protective layer composed of Polyvinylidene Fluoride (PVDF) conforming to (AAMA 307-16), Voluntary Specification for Laminates Intended for Use on AAMA Certified Profiles.

- Non acrylic based film products, or any film products containing PVC shall be allowed.

- Film Warranty Period:	20 years from date of manufacture consisting of a 10 year full replacement warranty with labor and materials included, and an additional 10 year prorated finish material only warranty.

- Color Selection:  Please select a color

- Exterior film shall cover all exposed exterior window frame and sash components.

2.07	FABRICATION

A.  Window frames and sashes shall be assembled in a secure and workmanlike 		      manner to perform as hereinafter specified.  All joints of main frames and 		      sashes shall be mitered and joined neatly, and secured by the means of 		      fusion welding.

PART 3           EXECUTION

3.01	 EXAMINATION

A.  Site Verification of Conditions:

1. 	Verify that building substrates permit installation of windows according to the manufacturer's instructions, approved shop drawings, calculations and contract documents.

2. 	Do not install windows until unsatisfactory conditions are corrected.

3.02	 INSTALLATION

A. Erection of Vinyl Windows

- Install windows with skilled tradesmen in exact accordance with approved Shop Drawings, Installation Instructions, Specifications, and in accordance with (AAMA  101/I.S.2./ A440).

- Windows must be installed plumb, square, and level for proper weathering and operation.  Jambs must not be “sprung”, bowed, or warped during installation.

B.   Field Tests

- Field testing procedure of installed windows shall be in accordance with AAMA 502.

- The test pressure used during the field test procedure shall be 2/3 of the rated test pressure of the test specimen in accordance with AAMA 502.  There shall be no optional variances over 2/3 used during testing.

- Initial field testing must be performed prior to no more than 5% of windows have been installed.  All field testing expenses shall be at the burden of the contractor.

3.02   ADJUSTING AND CLEANING

A.     After completion of window installation, windows shall be inspected, 	adjusted, put into working order and left clean, free of labels, dirt, or other 	debris.  Protection from this point shall be the responsibility of the General 	Contractor.

END OF SECTION